This manuscript experimentally investigates the deformation, strength and energy absorption of auxetic metastructure-reinforced granular composites under monotonic, constant shear stress and cyclic triaxial loading paths. It reveals the nonlinear enhancement of energy absorption and an efficiency crossover phenomenon between single-layer and triple-layer configurations, offering new experimental evidence for metastructure application in confined granular systems. However, the manuscript currently has several notable issues. Its core novelty is not sufficiently highlighted relative to conventional granular reinforcement. Some mechanistic explanations remain qualitative and lack in-depth discussion. Missing key experimental details undermines reproducibility. The manuscript also contains grammatical errors, non-standard terminology and formatting inconsistencies. I recommend a major revision based on the following constructive points.

- Line 34,45: There are typographical errors in the abstract. The “metastructure-embeded” should be corrected to “metastructure-embedded”, and “metastucture” should be corrected to “metastructure”. Please ensure consistent spelling throughout the manuscript.

- Lines 96–108: The research gap description is not sufficiently focused. While the authors point out that existing studies mostly focus on standalone metastructures and simplified uniaxial loading, they do not clearly elaborate the essential difference between auxetic metastructure reinforcement and conventional geotechnical reinforcement. Please clarify how the negative Poisson’s ratio behavior generates a self-adaptive confinement effect that cannot be achieved by traditional reinforcement.

- Line 112: The rationale for selecting the re-entrant topology as the representative metastructure is missing. Please briefly explain its advantages over other auxetic topologies (e.g., chiral structures) in granular confinement applications.

- Lines 162–163: Key geometric parameters of the metastructure unit cell (e.g., unit cell dimension, re-entrant angle, rib thickness) are only illustrated in Figure 2. (a) but not systematically listed in Table 2, resulting in information inconsistency between the figure and table. Please supplement these geometric parameters into Table 2 to align the information across figures and tables, and to ensure experimental reproducibility.

- Line 202: The pluviation method is adopted for specimen preparation, but key control parameters and the procedure to achieve 30% and 70% relative densities are not provided. Please supplement these details to ensure repeatability.

- Lines 213–217: Only the target Skempton B-value (≥ 0.95) is given. Please add the actual measured range of B-values to verify the saturation quality of specimens.

- Lines 240–242: Only one deviatoric stress level (250 kPa) and two confining pressures are used in CSS tests, which is relatively limited. Please explain the basis for selecting this deviatoric stress level.

- Line 327-328: There is a recurring typographical error within the graphs in Figure 8. The label “Oneset of instability” should be corrected to “Onset of instability”.

- Lines 354–358: The residual state line shifts toward higher void ratios and higher mean effective stresses with more reinforcement layers. Please further interpret the physical mechanism.

- Lines 408–410: Please provide the specific fitting equations of peak stress ratio vs. state parameter for each reinforcement condition to facilitate quantitative comparison.

- Lines 466–470: Only qualitative description is given that reinforcement delays pore pressure accumulation. Please add quantitative comparison of the number of cycles to instability for different configurations.

- Lines 546–552: The specific net energy index has a dimensionless numerator divided by mass, resulting in units of g-1, which has unclear physical meaning. Please explain the necessity of this normalization approach and its advantages over conventional specific energy absorption (J/kg). In addition, please provide the actual mass values of single-layer and triple-layer metastructures for transparency.

- Lines 560–578: The explanation of the efficiency crossover phenomenon relies on the hypothesis of “overlapping confinement influence zones”, which lacks direct experimental evidence. Please discuss how this hypothesis could be verified (e.g., via local deformation measurement or numerical simulation), and analyze the relationship between the crossover cycle number and confining pressure.